3DGPU

3D Graphics Processing Unit

Final Report

James Doty

Kyle McGhee

Allan Patterson

Daniel Wilhelm

ECE 495D

Tuesday, 2:30-5:20pm

TA: Philip Vorsilak

Submitted 04/29/05

___________________________  ___________________________

___________________________  ___________________________

1.0 EXECUTIVE SUMMARY

Graphics Processing Units (GPUs) have become standard components in modern-day computer systems, enabling users to render two- and three-dimensional graphics in parallel with the microprocessor. GPUs can render graphics efficiently due to the massive parallelization possible within the graphics pipeline, making it an ideal target for hardware speedup. Recently, several GPU manufacturers began developing low-power GPUs aimed at the burgeoning cell phone market, realizing increased demand for user interfaces, screen savers, and games.

A 3D Graphics Processing Unit (3DGPU) capable of rendering 3D points and lines to a low-resolution, low-color-depth graphics display will be implemented, similar to those on modern high-end cell phones and handheld gaming devices.

The design is important because it forms the basis for a complete pipelined GPU. Alone, it can render all possible 3D entities given an appropriate CPU algorithm, including meshes and filled polygons. The 3DGPU also incorporates a depth-buffer, allowing easy hidden surface removal. Future projects could easily extend this pipelined framework, incorporating support for more advanced primitives and algorithms, as well as the parallelization support necessary to compete with modern cards.

To develop the 3DGPU, four engineers are required for six weeks. VHDL will be used to implement high-level pipeline stages such as 3D projection and line rendering, and C code will be written to test the design by simulating an LCD display panel. Since the design is very modular, the modules will be split between the engineers for initial development and testing.

This document contains the proposal for the 3DGPU project. In Section 2, general design specifications are presented, along with the available opcodes that the processor and modules will support. Section 3 contains a top-level block diagram of the 3DGPU, illustrating input and output pins along with output signals. Section 4 illustrates the projected milestones and task assignments amongst the engineers, and Section 5 concludes the proposal by giving criteria that the final product can be tested on that ensure success.

2.1 INTRODUCTION
	Graphics Processing Units (GPUs) are microprocessors which are optimized for the display of 3D graphics and 2D rasterization by parallelizing each block in the graphics pipeline in hardware. As computers have undergone miniaturization, now being available in cell phones and handheld videogame consoles, consumers have demanded more features in smaller areas. Companies such as nVidia and ATI have recently responded to this demand by developing low-power GPUs which provide 2D and 3D acceleration to these devices while dealing with constraints such as reduced functionality, fewer pins, and lower power dissipation.

The purpose of the 3DGPU project is to develop a bare-bones GPU designed for embedded devices, such as cell phones, that is capable of accelerating 2D and 3D graphics rendering. The project also will provide advanced features such as page flipping to reduce flicker, and z-buffering for hidden surface removal. From the user’s perspective, all 2D and 3D operations will be rendered faster on their display. If the 3DGPU is programmed correctly, the user will notice that flicker does not occur, and that pixels on the screen are ordered by their depth away from the user, such that closer objects occlude farther objects.

The 3DGPU application is a hand-held device designed to operate from a low voltage power source. The primary special requirements for the 3DGPU are power dissipation and pin count since the final aim for the device is to be eventually used for larger next-generation high-end cell phones and low-end handheld videogame consoles. Since miniaturization of transistors yields more power leakage, more powerful cooling devices are required as more power is leaked. Pin count is also an important factor because for each pin, the output logic for that pin must be localized next to it; thus, the more pins used, fewer area optimizations are possible, and also the distance between each pin is reduced, increasing the possibility of leakage between pins.

The primary optimization objective of the 3DGPU is speed. GPUs are often measured by the number of elements rendered per unit time, and this is directly dependent on speed and the algorithms used. For instance, the 3DGPU will thus require an efficient parallelized line algorithm since that module is the largest bottleneck in the pipeline. More importantly, since the 3DGPU will serve as the foundation for more advanced GPUs, it must be as fast and transparent a layer as possible.

Technical Features

- Architecture allows for 8-bit color 3D line and pixel plotting on a 256x256 LCD display.

- Depth ordering per pixel implemented through 8-bit w-buffer.

- Perspective projection applied to all 3D points for realistic depth display.

- Page flipping architecture allows for no flicker in display rendering.

- Pass-by-address utilized for opcodes, allowing large data structures (e.g. 64-byte matrices) to be referenced using a single 2-byte value.

- Three separate SRAM modules allow computer, LCD display, and controller to access data simultaneously without long wait times.

- Variable frequency for display output.

- Line algorithm adapted for efficient hardware implementation.

- Use new 16.16 signed fixed point values for all internal fractional data.

- Modifiable field of view.

Controller

Matrix Math

3D Projection

X FOV (latched)

Y FOV (latched)

Line Rasterization

Screen Buffer

currColor (latched)

currPage (latched)

Shift Register

Opcode (latched)

Input Data (latched)

LCD Controller

LCD Driver

(external)

Implemented in C

Graphics Pipeline (Periphery)

Pixel Output

Matrix/3D Vector

3D Point

Two 2D Points + Z

2D Point + Z

Addr+Color+Depth

Input Data

FIGURE 1.

3DGPU high-level block diagram.

3DGPU

SRAM

Video Buffer II

8-Bit Color +

8-Bit Depth

SRAM

Video Buffer I

8-Bit Color +

8-Bit Depth

LCD_CLK

CPU_CLK

Addr+Color+Depth

currMatrix (latched)

CPU INPUT

Opcode

data

SRAM Interface

2D Point + Z

Flip

Division

2.2 DESIGN CONSIDERATIONS

3DGPU System Inputs (from the CPU)

A CPU exists externally to the GPU. The CPU can read and write directly to any area of the SRAM. An output signal from the SRAM module (mem_wait) is high when the memory is being accessed. Any device requesting to read or write to SRAM must wait until mem_wait is cleared before proceeding to ensure data integrity. The CPU also sends an assembly instruction to the 3DGPU consisting of a 4-bit opcode on OPCODE and a 32-bit data field on DATA which can contain addresses or values which express 16.16 signed fixed-point or integral values. Fed in by singular pins from the CPU, there is a load ‘handshaking’ pin, LOAD, which notifies when to load an instruction into the controller, as well as a clock pin CLK and an asynchronous reset pin nRST.

3DGPU System Outputs (to the LCD Display Driver)

The GPU uses these inputs to drive the eight-bit COLOR bus which connects to an LCD Driver Chip whose output routes to an LCD Display. Each 8-bit pixel color value is outputted from LCD_Controller, starting at the upper-left pixel of the display, proceeding to the end of a scan line, then down to the lower-left of the next row, while asserting the ROW output pin.  These outputs are only driven when the ENABLE1 signal is fed into the system.

3DGPU Data Formats

The 3DGPU contains several important data formats, illustrated in Table 1.

3DGPU Operations

Each of the 3DGPU operations consists of a series of control signal (*_OPCODE) transfers from Controller to each Module (Tables 2 and 3), along with an accompanying *_ENABLE signal to the appropriate module. Upon completion, each module lowers a *_BUSY signal back to the Controller, notifying that the next operation can be commenced.

‘setmat’ – The controller module shifts in a matrix from the CPU through the shift register and places it into MM0_DATA and MM1_DATA.  This is passed into the two matrix_math modules on DATA.  This matrix is then stored locally in both matrix_math modules so that future operations may be performed on it.

‘mulmat’ – The controller module shifts in a matrix from memory through the shift register and places it into MM0_DATA and MM1_DATA.  This matrix is passed into the two matrix_math modules on DATA.  Both matrix_math modules then multiply the locally stored matrix by the input matrix, and the result is stored in the local matrix in each matrix_math module.  The formula for this operation can be seen in section 5, where the proper functioning of this method is listed as one of the success criteria.

‘rstmat’ – The controller module accesses both matrix_math modules, and each matrix_math loads the current stored matrix with the identity matrix with the main diagonal having values of ‘1’ and all other elements having values of ‘0’. That is,

1	0	0	0

0	1	0	0

0	0	1	0

0	0	0	1

‘xfov’ – The controller module passes a value through TP1_DATA and TP2_DATA to both of the 3D_Projection modules into their DATA pins. Each TD_projection module then stores the value, by flip-flopping, in the module as the new x field-of-view.

‘yfov’ - The controller module passes an value through TP1_DATA and TP2_DATA to both of the 3D_Projection modules into their DATA pins. Each TD_projection module then stores the value, by latching, in the module as the new y field-of-view.

‘pt3d’ – The controller loads a vector from the shift register and passes it to one of the matrix_math modules via DATA.  The matrix_math module then multiplies the vector using the stored matrix.  The projected vector is passed by RESULT into the corresponding TD_projection module on DATA.  The input x and y value is then divided by the FOV constants, and then divided by the z value.  The result is a real number between 1 and -1 which is then scaled by adding one to eliminate the negative and multiplying by 128, yielding a value between zero and 255. This scaled value is outputted by RESULT to screen_buffer through DATA. The screen_buffer then requests the point be drawn to the appropriate SRAM (video page) through SRAM_controller.  This point is later retrieved when the display memory pages are flipped, and the LCD_Controller then draws the point.

‘ln3d’ – The controller module receives two vectors as input in DATA.  The controller then passes each one to a separate matrix_math module via DATA.  Each matrix_math module then multiplies the vector using the stored matrix.  The projected vectors are then passed by RESULT into the corresponding TD_projection modules on the respective DATA.  The input x and y values are then scaled in TD_projection to values on the display.  These scaled values are output by RESULT to the controller then back to Line_Rasterization through DATA. Line_Rasterization then generates a series of interpolated 2D points which are passed into screen_buffer.  The screen_buffer module then draws the point to the appropriate SRAM (video page) through SRAM_controller.  This point is later retrieved when the display memory pages are flipped, then the LCD_Controller draws the point.  Control is returned to the controller when the Line_Rasterization module is completed, and its BUSY signal is set low back to the controller’s LR_BUSY.

‘color’ – The controller module sets the color value to SB_DATA and passes it into screen_buffer on DATA.  The screen_buffer module locally stores the value by latching it.

‘clr’ – The controller module passes a control signal to screen_buffer.  Screen_buffer uses the current latched color to write to the SRAM_controller the current video page in SRAM, and it fills the entire current video buffer with that color.

‘flip’ – The controller module passes a flag, LCD_FLIP, to LCD_Controller on FLIP.  At this point, LCD_Controller finishes drawing the current page and then stops drawing. It responds by asserting FLIPPED back to the controller’s LCD_FLIPPED.  Once this handshaking is done, LCD_Controller sets an internal latched value to tell it which SRAM it will be drawing off of now (the opposite of whatever it previously was), and resumes drawing.  Finally, the controller module sends an opcode to screen_buffer via OPCODE.  Screen_buffer then sends a notice to the SRAM_controller to toggle between the active video page SRAM and the inactive video page SRAM by inverting a locally stored variable.

‘pt2d’ – A 2D vector is loaded into the controller via DATA.  This value is passed into screen_buffer by SB_DATA.  The screen_buffer module then sets the appropriate pixel address to the SRAM_controller on the active SRAM to the current color through DataBus1 or DataBus2.

‘ln2d’ – Two 2D vectors are loaded into the Controller via DATA.  These vectors are then loaded by LR_DATA, into Line_Rasterization at DATA.  Line_Rasterization then proceeds to calculate a series of interpolated 2D points, and passes them by RESULT to screen_buffer by DATA.  The screen_buffer module then draws the point to the appropriate SRAM (video page).  This point is later retrieved when the display memory pages are flipped, then the LCD_Controller draws the points.  Control is returned to the controller when the Line_Rasterization module is completed, and its BUSY signal is set low back to the controller’s LR_BUSY.

| Data Type | Description | Bytes | C data storage type |
| --- | --- | --- | --- |
| INT | Integer input from CPU | 4 | unsigned long |
| FIXED | Fixed Point (IEEE) to store internal data | 4 | unsigned long |
| COLOR | Color to store color values | 1 | unsigned char |
| DEPTH | Z depth value of pixel | 1 | unsigned char |
| ADDR | Absolute SRAM Address input from CPU | 2 | short int |
| 2DVEC | 2D Vector fixed point values | 8 | struct 2DVEC { FIXED x, y; }; |
| 3DVEC | 3D Vector fixed point values | 12 | struct 3DVEC { FIXED x, y, z; }; |
| MATRIX | 4x4 Matrix fixed point values | 64 | struct MATRIX { FIXED x1, y1, z1, w1; FIXED x2, y2, z2, w2; FIXED x3, y3, z3, w3; FIXED x4, y4, z4, w4; }; |

TABLE 1. Data Formats Used

| Command Signal | Matrix Math | 3D Projection | Line Rasterization | Screen Buffer |
| --- | --- | --- | --- | --- |
| 000 | nop | nop | nop | no draw |
| 001 | vector multiply | project | rasterize | set color |
| 010 | store in SRAM | set X FOV |  | plot pixel |
| 011 | store identity | set Y FOV |  | clear buffer |
| 100 | Matrix multiply and store |  |  | page flip |
| 101 |  |  |  |  |
| 110 |  |  |  |  |
| 111 |  |  |  |  |

TABLE 2. Module Control Codes. 
NOTE: Control module does nothing if undefined.

| Opcode | Mnemonic | Matrix Math | 3D Projection | Line Rasterization | Screen Buffer |
| --- | --- | --- | --- | --- | --- |
| 0000 | setmat | 010 |  |  |  |
| 0001 | mulmat | 100 |  |  |  |
| 0010 | rstmat | 011 |  |  |  |
| 0011 | xfov |  | 010 |  |  |
| 0100 | yfov |  | 011 |  |  |
| 0101 | pt3d | 001 | 001 |  | 010 |
| 0110 | ln3d | 001 | 001 | 001 | 010 |
| 0111 | color |  |  |  | 001 |
| 1000 | clr |  |  |  | 011 |
| 1001 | flip |  |  |  | 100 |
| 1010 | pt2d |  |  |  | 010 |
| 1011 | ln2d |  |  | 001 | 010 |
| 1100 | UNUSED |  |  |  |  |
| 1101 | UNUSED |  |  |  |  |
| 1110 | UNUSED |  |  |  |  |
| 1111 | UNUSED |  |  |  |  |

TABLE 3. Control Code Transition Table By Opcode
NOTE: Control module does nothing if undefined.

| OPCODE | MNEMONIC | DESCRIPTION |
| --- | --- | --- |
| 0000 | setmat addr_MATRIX | Set Current Matrix. |
| 0001 | mulmat addr_MATRIX | Multiply Current Matrix. |
| 0010 | rstmat | Reset Current Matrix to Identity. |
| 0011 | xfov val_FIXED | Set X Field of View. |
| 0100 | yfov val_FIXED | Set Y Field of View. |
| 0101 | pt3d addr_3DVEC | Draw 3D point. |
| 0110 | ln3d addr_3DVEC addr_3DVEC | Draw 3D line. |
| 0111 | color val_COLOR | Set Color. |
| 1000 | clr | Clear Buffer. |
| 1001 | flip | Page Flip. |
| 1010 | pt2d addr_2DVEC | Draw 2D point. |
| 1011 | ln2d addr_2DVEC addr_2DVEC | Draw 2D line. |
| 1100 | XXX | UNUSED |
| 1101 | XXX | UNUSED |
| 1110 | XXX | UNUSED |
| 1111 | XXX | UNUSED |

TABLE 4. Supported Feature Set
NOTE: Defaults to ‘No Operation’ if undefined.

NOTE: ‘addr_{TYPE}’ indicates that an absolute address is passed from the
controller that points to the type TYPE in SRAM.

‘var_{TYPE}’ indicates that a value of type TYPE is passed from the controller

3.0 FINAL DESIGN

Top Level Design

The entire system works by inputting opcodes, data/addresses, and a load signal from a CPU, and running through each opcode, and finally calculating outputs for an LCD driver.

Inputs: 	OPCODE (4 bit) 4 pins to determine current opcode

DATA1 (32 bit) Address/Data line in

LOAD (1 bit) “handshaking” LOAD signal

ENABLE (1 bit) Main chip enable

ENABLE1 (1 bit) LCD_Controller enable

Outputs: 	COLOR (8 bit) LCD display signal

ROW (1 bit) Row completion flag

BUSY6 (1 bit) Busy signal to CPU

Controller

Design: State Machine

This module receives the current opcode and “runs” the instructions by sending control instructions to each necessary block in the correct order along with the corresponding ENABLE signal. The controller knows when a certain process in the chain is finished by watching for the lowering of the respective BUSY signal.

Inputs: 	OPCODE (4 bit) Opcode input

DATA (32 bit) Multipurpose data input

LOAD (1 bit) Load instruction signal

ENABLE (1 bit) Chip Enable for waiting for preprocessing to SRAM to complete.

MM_BUSY, TP_BUSY, LR_BUSY, SB_BUSY, (4 * 1 bit) Busy signal from each block

LCD_FLIPPED (1 bit) ‘done’ signal from LCD_Controller when it has flipped video pages (SRAM).

CLK (1 bit) Clock (Input to all blocks - NOT SHOWN ON DIAGRAM FOR SIMPLICITY)

nRST (1bit) Asynchronous Reset (Input to all blocks - ALSO NOT SHOWN ON DIAGRAM)

memWait (1 bit) Informs whether the SRAM is busy or not

Bidirectional: DataBus (16 bits) Data bus to SRAM

Outputs:      MM1_OPCODE, MM0_OPCODE (2*2 bits) Control signals to each matrix_math module

TP1_OPCODE, TP0_OPCODE (2*4 bits) Control signals to each TD_Projection module

LR_OPCODE (2 bits) Control signal to Line_Rasterization module.

SB_OPCODE (2 bits) Control signal to screen_buffer module

MM1_DATA, MM0_DATA (2*512 bits) Data signals to each matrix_math module (capable of loading a 4*4 * 32 bit matrix)

TP1_DATA, TP0_DATA (2*96 bits) Data signals to each TD_Projection module (capable of loading a 3 * 32 bit vector)

LR_DATA (48 bits) Data signal to Line_Rasterization module (capable of holding (2 * (3 * 8 bit) points))

SB_DATA (32 bits) Data signal to screen_buffer module

SB_ENABLE (1 bit) enable signal to trigger screen_buffer operation

LCD_FLIP (1 bit) enable signal to trigger LCD_Controller page flipping

nOutputEnable (1 bit) Active low output enable to SRAM				nWriteEnable (1 bit) Active low write enable to SRAM

nChipEnable (1 bit) Active low chip enable to SRAM

Address (16 bits) Address bus to SRAM

DumpSignal (1 bit) Signal to dump SRAM information to a text file

BUSY (1 bit) Output signal to CPU to inform when ready for next 				instruction

matrix_math

Design: Procedural Arithmetic

This module multiplies matrices together, multiplies vertices by matrices, and stores the current matrix.

Number Implemented: 2

Controls:	Set current matrix (matrix from controller)

Multiply current matrix (matrix from controller)

Transform vertex (X,Y,Z from controller)

Reset to identity

Inputs:	DATA (512 bits) Holds either Vertexes for transformation, or 	matricies for setting or multiplying

ENABLE (1 bit) tells the module to begin operations

OPCODE (2 bits) specific operation instructions

memWait (1 bit) Informs whether the SRAM is busy or not

CLK (1 bit) Clock (Input to all blocks - NOT SHOWN ON DIAGRAM FOR SIMPLICITY)

nRST (1bit) Asynchronous Reset (Input to all blocks - ALSO NOT SHOWN ON DIAGRAM)

Outputs:	RESULT (3*32 = 96 bits) Transformed vertex

BUSY (1 bit) tells controller when a job is completed

3D_Projection

Design: Procedural Arithmetic

This module takes a vertex from the Matrix_Math module and projects it onto a 2D display.

Code: (x,y) = (x/(z*FOVx) + 1, y / (z * FOVy) + 1) * Resolution

Number Implemented: 2

Controls:	Project point

Inputs:	DATA (3*32 = 96 bits) 3D Vertex or field of view

ENABLE (1 bit) tells the module to begin operations

OPCPDE (4 bits) specific operation instructions

CLK (1 bit) Clock (Input to all blocks - NOT SHOWN ON DIAGRAM FOR SIMPLICITY)

nRST (1bit) Asynchronous Reset (Input to all blocks - ALSO NOT SHOWN ON DIAGRAM)

Outputs:	RESULT (8*3 = 24 bits) 2D Vertex with scaled depth

BUSY (1 bit) tells controller when a job is completed

Line_Rasterization

Design: State Machine

This module interpolates each point on a line, using bilinear interpolation, and clips the line to fit the screen dimensions. The module also remembers the last two points sent to it and uses those to calculate a line.

Controls:	Get points (gets a point from 3d projector)

Calculate line and pass points to buffer (BUSY is set low when line has been completely drawn, otherwise BUSY is held high and this command needs to be performed again.)

Pass single point to Screen Buffer

Algorithm:  First, the line must be computed at every pixel value along the y axis or every value along the x axis between two points to retain line continuity. The y values are incremented by one each cycle if the y/x slope > 1, else the x values are incremented by one each cycle. Further, since the slope of the other two coordinates (other than the one being incremented) are incremented by a constant (fractional) amount each cycle, then we add the slope of the line to the current other two coordinate values to yield the new position of the line (after rounding to the nearest integer). This is continued until the constantly incrementing x or y value reaches the ending line point. Note that (x1,y1,z1)->(x2,y2,z2) is equivalent to its converse given no special color assumptions.

Inputs:	DATA (2*24 = 48 bits) 2D Vertexes to interpolate

ENABLE (1 bit) tells the module to begin operations

OPCODE (2 bits) specific operation instructions

SB_BUSY (1 bit) comes from screen_buffer to establish when a draw is completed.

CLK (1 bit) Clock (Input to all blocks - NOT SHOWN ON DIAGRAM FOR SIMPLICITY)

nRST (1bit) Asynchronous Reset (Input to all blocks - ALSO NOT SHOWN ON DIAGRAM)

Outputs:	RESULT (32 bits) Individual interpolated 2D points on line (w/ 	depth)

SB_ENABLE (1 bit) tells Screen_Buffer to draw the output point

BUSY (1 bit) tells controller when a job is completed

screen_buffer

Design: State Machine

This module interfaces with SRAM_Controller to maintain two screen memory buffers, an active buffer and an inactive buffer.  The screen buffer writes points with a depth and a color to the SRAM.

Controls:      Set pixel in current draw buffer (x, y, depth)

Swap buffer (no inputs)

Clear current draw buffer (color to black, depth to infinity) (no 				inputs)

Inputs:	DATA (32 bits) Pixel to set in SRAM, or color to store in module

ENABLE (1 bit) tells the module to begin operations

OPCODE (2 bits) specific operation instructions

memWait1, memWait2 (2*1 bit) Informs whether the according SRAM is busy or not

CLK (1 bit) Clock (Input to all blocks - NOT SHOWN ON DIAGRAM FOR SIMPLICITY)

nRST (1bit) Asynchronous Reset (Input to all blocks - ALSO NOT SHOWN ON DIAGRAM)

Bidirectional: DataBus1, DataBus2 (2*16 bits) Data bus to SRAMV1 and 					SRAMV2, accordingly

Outputs:	nOutputEnable1 (1 bit) Active low output enable to SRAM				nWriteEnable1 (1 bit) Active low write enable to SRAM

nChipEnable1 (1 bit) Active low chip enable to SRAM

Address1 (16 bits) Address bus to SRAM

DumpSignal1 (1 bit) Signal to dump SRAM information to a text file

nOutputEnable2 (1 bit) Active low output enable to SRAM				nWriteEnable2 (1 bit) Active low write enable to SRAM

nChipEnable2 (1 bit) Active low chip enable to SRAM

Address2 (16 bits) Address bus to SRAM

DumpSignal2 (1 bit) Signal to dump SRAM information to a text file

BUSY (1 bit) tells controller that current job has been completed

LCD_Controller

Design: Independent module

This module interfaces with SRAM to read from one of two screen memory buffers, an active buffer and an inactive buffer.  LCD_Controller uses screen_buffer’s “inactive” buffer, and continuously draws the contents of the buffer to an external LCD driver chip.  Essentially, this module is independent of all other modules, and is added into the chip to serve as a handshaking component to a standard LCD driver.  This module can be interrupted by the controller, to trigger a page flip, and otherwise just continuously reads from the appropriate SRAM.

Controls:     Read point from SRAM, output to LCD Driver

Swap buffer

Inputs:	FLIP (1 bit) tells the module to halt drawing after the current pass, 	and then switch buffers

memWait1, memWait2 (2*1 bit) Informs whether the according SRAM is busy or not

nRST (1bit) Asynchronous Reset (Input to all blocks - ALSO NOT SHOWN ON DIAGRAM)

Bidirectional: DataBus1, DataBus2 (2*16 bits) Data bus to SRAMV1 and 					SRAMV2, accordingly

Outputs:	FLIPPED (1 bit) tells controller when the video page is flipped to the 	other SRAM

nOutputEnable1 (1 bit) Active low output enable to SRAM				nWriteEnable1 (1 bit) Active low write enable to SRAM

nChipEnable1 (1 bit) Active low chip enable to SRAM

Address1 (16 bits) Address bus to SRAM

DumpSignal1 (1 bit) Signal to dump SRAM information to a text file

nOutputEnable2 (1 bit) Active low output enable to SRAM				nWriteEnable2 (1 bit) Active low write enable to SRAM

nChipEnable2 (1 bit) Active low chip enable to SRAM

Address2 (16 bits) Address bus to SRAM

DumpSignal2 (1 bit) Signal to dump SRAM information to a text file

Signed Divide Module

Design: Strictly dataflow.

Divides 16.16 signed (two’s complement) fixed point numbers to give a 16.16 signed result.

Controls: None (not affected by controller)

Inputs: X (32 bits) 16.16 signed fixed point numerator

Y (32 bits) 16.16 signed fixed point denominator

Outputs: Z (32 bits) 16.16 signed fixed point quotient

V (1 bit) active high overflow bit

Shift Register Module

Design: Independent module, state machine.

Incoming data from the CPU enters through OPCODE_IN and DATA_IN, where it is stored in a queue for retrieval by the GPU when it is ready for a new signal.

Controls: None (pre-Controller).

Inputs: nRST (1 bit) Active-low reset.

READ (1 bit) Whether a value is to be shifted out.

LOAD (1 bit) Whether a value is to be loaded from DATA_IN

OPCODE_IN (4 bits) The opcode entering the GPU

DATA_IN (32 bits) The data entering the GPU, associated with the opcode

CLK (1 bit) The global clock)

Outputs: OPCODE_OUT (4 bits) Opcode sent to the Controller

DATA_OUT (512 bits) The entire output of the register

BUSY_OUT (1 bit) Whether a read is inadvisable

SRAM, SRAM_V1, SRAM_V2

This module is an SRAM module from the ECE437L website.  The module operates as readable/writable memory which can be accessed by other modules and external devices, namely, the system CPU.

Number Implemented: 3 (2 Video page buffers, one general data storage)

Inputs:	nOutputEnable (1 bit) Places data at Address on DataBus

nWriteEnable (1 bit) Places data on DataBus at Address

Address (16 bits) current memory address

nChipEnable (1 bit) Active low chip enable to SRAM

DumpSignal (1 bit) Signal to dump SRAM information to a text file

Bidirectional: DataBus (16 bits) (contains data to be written or read from 					Address)

Outputs:	memWait (1 bit) States if SRAM is in use

3.1 PIN DESCRIPTION

The pins of this device can be divided into two major categories – preprocess signals and run-time signals.  This is largely due to the operation practice of the GPU design.  Many frequently used values must be loaded into GPU memory before operation to avoid bus fighting issues and maximize speed.  They are tabulated as such.

| GPU SRAM Preprocess Signals |  |  |
| --- | --- | --- |
| Signal Name | Direction | Description |
| nWriteEnable | IN | This signal causes the SRAM module to write the data present on the DataBus into memory. |
| DataBus[15:0] | IN | The data port to the SRAM module. This value is written to the SRAM at the specified Address. |
| Address[15:0] | IN | The location in SRAM to write to. |
| memWait | OUT | This signal is a feedback signal to the SRAM user. When this signal is high, all inputs to the SRAM should be held. It’s much like a “working” indicator. If any signals are changed while this signal is high, unexpected results may occur (writes may not store data). |

| GPU Run-Time Signals |  |  |
| --- | --- | --- |
| Signal Name | Direction | Description |
| CLK | IN | The GPU System Clock. |
| LCD_CLK | IN | The LCD display output Clock. This signal is not necessarily synchronous with the system clock, and determines the frequency of the output color and row signals. It is rising edge sensitive. |
| nRST | IN | The GPU System Reset. When this signal is asserted-low (logic ‘0’), the registered elements within the GPU are set to their default values. |
| ENABLE | IN | The GPU chip enable. Unless this signal is asserted-high (logic ‘1’), the chip will not operate. While in a logic ‘0’ state, all SRAM preprocessing should occur. |
| LCD_ENABLE | IN | Tthe LCD display output enable. Unless this signal is asserted-high (logic ‘1’), no signals will be output on either COLOR or ROW. |
| LOAD | IN | When this signal is synchronously pulsed from low (logic ‘0’) to high (logic ‘1’), the current opcode on the OPCODE signal is read in, and processed. This allows the processor communicating with the GPU to operate at a slower clock speed than the GPU, and ensures that an opcode will not be processed multiple times. |
| OPCODE[3:0] | IN | The current opcode instruction being transmitted to the GPU from another processor. Its value (see table 4) determines the action of the GPU, and what bits of DATA are used. |
| DATA[31:0] | IN | The data necessary for the current OPCODE (see table 4). Values are right justified – that is, in some cases not all 32 pins are used, and in these cases the lower pins are used (for example, if inputting a 16 bit address, the address should be written on pins 15 down to 0). |
| BUSY | OUT | The GPU system busy signal. While this flag is asserted-high (logic ‘1’), the GPU is processing the previous opcode, and will not read in any additional values until it finishes. If it is low (logic ‘0’), and at least one GPU clock cycle has passed, the GPU is ready for another input. |
| COLOR[7:0] | OUT | This is the color value being output on the LCD_CLK signal. It outputs the value of each pixel one at a time in horizontal scan-line order. |
| ROW | OUT | This signal will be asserted for one LCD_CLK cycle at the first color of each row being output by COLOR. |

3.2 TIMING & AREA BUDGETS

See next few pages. Note that the TOTAL core area estimated was 27423420 um^2, and the actual synthesized area is 30891171 um^2. This is an 11% difference. Also note that the timing analysis was not possible both due to the complexity of the design and the busyness of the servers.

3.3 EXTERNAL INTERFACE

The designed GPU will interface between some controlling processor (described below) and a LCD display driver.  The controlling processor determines what actions are to be calculated and performed on the GPU, drawing a picture, and the GPU will take that information and output an appropriate signal to the LCD driver.  The output signal is very similar to many LCD manufacturer standards.

Block Diagram

The Controlling Processor Interface

Data is passed into the GPU from an external pin which is connected to a CPU’s data output. On OPCODE, a 4-bit value is inputted which specifies which action the controller is to take. On DATA, a 32-bit value is inputted which specifies a value which is interpreted either as a 16.16 signed (two’s complement) fixed point or in standard integral format, depending on the signal on OPCODE. At most, a single opcode will require the shifting-in of sixteen 32-bit DATA signals to accommodate for a matrix of sixteen 16.16 values.

The LCD Driver Interface

The interface with the LCD driver is extremely fundamental.  The output signal can be considered almost entirely independent of the GPU.  With each LCD_CLK rising edge, a pixel color is output.  The GPU will cycle through every single pixel in the screen (in this case, all 256x256 or 65536 of them) outputting to COLOR, until all are complete, then it will simply repeat the process.  Every 256 pixels (or, equivalently, at the end of every row), ROW will be asserted high for a single LCD_CLK pulse, and is otherwise low.  See Figure 2 for illustration of these signals.

LCD_CLK

COLOR

ROW

Pixel #

0

1

...

255

Figure 2. LCD Driver Signals

Interface Emulation

Two interfaces were emulated in the design and testing of the GPU. First, a software LCD emulation tool was created that took signals generated in simulation by ModelSim and converted them into a visual LCD panel of 256x256 colors. The 8-bit color was interpreted as grayscale. To retrieve the necessary data from ModelSim, the Lists feature was used which outputs an ASCII data file of any signals of the user’s choice. This list was then inputted into the emulated LCD interface to render the display.

The second interface was the emulation of the CPU. An assembler was authored that inputs an assembly file using the opcodes described above and outputs a testbench which emulates the shifting of data into the GPU. Further, some instructions in the assembler were macros which expanded into more complex expressions for ease of writing.

| Storage Variable | Bytes | Initial Value Upon Reset | Location |
| --- | --- | --- | --- |
| X FOV FIXED | 4 | 1 (note: 90 degree FOV) | 3D Projection |
| Y FOV FIXED | 4 | 1 (note: 90 degree FOV) | 3D Projection |
| currColor COLOR | 1 | 0 (black) | Screen Buffer |
| currMatrix MATRIX | 64 | Identity | Matrix Math |
| Video Page 1 COLOR+DEPTH | 256x256x2 | Reset to 0s | SRAM |
| Video Page 2 COLOR+DEPTH | 256x256x2 | Reset to 0s | SRAM |
| currPage STD_LOGIC | 1 bit | 0 | Screen Buffer |

TABLE 5. Stored/Initial Values

4. Testing

Since the design was so large, testing was difficult at times. Most of the problems encountered were due to sign errors and data formats transmitted between blocks. Once each individual block appeared to be working on its own, we put them together. To test everything, we created a test bench generator and an output renderer. The test bench generator turns a text file containing assembly-type instructions regarding what to draw into a test bench which executes the instructions. This enabled testing a greater array of functionality with less effort. For instance, our final 3D test bench that drew a cube and rotated it more than 90 degrees was over 17,000 lines long. We never could have created this by hand. On the output side, we created a renderer that read in the signals that would go to an LCD device and would convert each frame to a jpeg image. This enabled us to see our errors visually. At one point our lines were not drawing to the correct points in space. By having the visual output we could easily see that our cube was not cube shaped. If all we had were waveforms we would have had a hard time seeing details like this.

Another technique we used to test problems with our synthesized design was replacing the source version blocks one by one with a synthesized block. If adding the new block broke the chip, we knew there was a problem with that block and could focus in on finding the problem better than if we had just tested the whole thing synthesized. Each individual was responsible for several modules, and when a problem was classified and duplicated within a testbench, then the original module author, knowing the module the best, would diagnose and solve the problem. The problem was translated into a testbench due to the immense amounts of time required for simulation – the design was large, and this act of specification helped to better classify the problem.

Since the design was heavily math-based, our drawing problems were solved by running the calculations by hand and checking the inputs and results from each block. This enabled us to catch sign errors and bit ordering errors fairly easily. Further, classifying errors as being within the hardware itself or the software which generated the test bench is an example of another problem faced. Ultimately, it was discovered that a figure was drawn incorrectly on the simulated LCD because the compiler only outputted ten lines. These mistakes were found by tediously scanning through long test benches to verify accuracy.

Block-level testing was accomplished by individual authors whereas chip-level testing was undertaken by everyone. To construct test benches, authors tried to enumerate examples which would cover every case of the algorithm employed. Sometimes, these cases were hard to enumerate so random examples could be chosen. Corner cases such as divide by zero were also important. For instance, divide by zero returns unexpected results in many scenarios, so a way was needed to avoid the problem or correct for it by returning a valid result. Further, points could be drawn off-screen, and to accommodate for this those points were drawn at (0,0) for easy debugging and handling.  See Appendix C for screenshots of important test benches, and see Appendix F for the test bench source code for each module.

5. Layout

Below is a table of the generated layout, followed by snapshots of the layouts.

|  | Aspect Ratio | Row Util | Dim (um) | Area (um²) | # gates |
| --- | --- | --- | --- | --- | --- |
| Controller | 1.00 | 75% | 571x 571 | 3262684 | 3844 |
| LCD_Controller | 1.00 | 85% | 419x 419 | 1759970 | 2025 |
| Line_Rasterization | 1.00 | 85% | 1362x 1362 | 18556978 | 23716 |
| Matrix_Math | 1.00 | 75% | 4323x 4323 | 186872915 | 246016 |
| Screen_Buffer | 1.00 | 85% | 419x 419 | 1283309 | 2025 |
| Shift_Register | 1.00 | 85% | 419x 419 | 1759970 | 2025 |
| SRAM_Control | 1.00 | 85% | 131x 131 | 172975 | 121 |
| TD_Projection | 1.00 | 85% | 1308x 1308 | 17121199 | 21609 |

TABLE 6
Generated layout statistics.

Controller IC

LCD Controller IC

Line Rasterization IC

Matrix Math IC

Screen Buffer IC

Shift Register IC

SRAM Control IC

TD Projection IC

5. Success Criteria

Fixed criteria:

1. (2 points) Test benches exist for all top level components and the entire design. The top level test benches can be demonstrated or documented to cover all of the functional requirements given in the design specific success criteria.

2. (4 points) Entire design synthesizes completely, without any inferred latches, timing arcs, or sensitivity list warnings.

3. (2 points) Source and mapped version of the complete design behave the same for all test cases

4. (2 points) A complete IC layout is produced that passes all geometry and connectivity checks.

Personal Criteria:

1. (2 points) Final mapped version interfaces with an external program to actually draw the output of the GPU, in this case, a 2D image.

2. (2 points) Demonstrate by simulation of a VHDL test bench that the GPU successfully implements in-memory matrix operations.  These operations primarily include multiplication of a 4x4 matrix, verifiable by using the built-in DUMP feature of the SRAM, and shown in the following formulas:

M11 M12 M13 M14		A11 A12 A13 A14

M21 M22 M23 M24    *    	A21 A22 A23 A24    =

M31 M32 M33 M34		A31 A32 A33 A34

M41 M42 M43 M44		A41 A42 A43 A44

M11new = M11*A11 + M12*A21 + M13*A31 + M14*A41

M12new = M11*A12 + M12*A22 + M13*A32 + M14*A42

M13new = M11*A13 + M12*A23 + M13*A33 + M14*A43

M14new = M11*A14 + M12*A24 + M13*A34 + M14*A44

M21new = M21*A11 + M22*A21 + M23*A31 + M24*A41

M22new = M21*A12 + M22*A22 + M23*A32 + M24*A42

M23new = M21*A13 + M22*A23 + M23*A33 + M24*A43

M24new = M21*A14 + M22*A24 + M23*A34 + M24*A44

M31new = M31*A11 + M32*A21 + M33*A31 + M34*A41

M32new = M31*A12 + M32*A22 + M33*A32 + M34*A42

M33new = M31*A13 + M32*A23 + M33*A33 + M34*A43

M34new = M31*A14 + M32*A24 + M33*A34 + M34*A44

M41new = M41*A11 + M42*A21 + M43*A31 + M44*A41

M42new = M41*A12 + M42*A22 + M43*A32 + M44*A42

M43new = M41*A13 + M42*A23 + M43*A33 + M44*A43

M44new = M41*A14 + M42*A24 + M43*A34 + M44*A44

Where MXYnew replaces MXY

3. (4 points) Demonstrate by simulation of a VHDL test bench that the output of GPU generates a meaningful image, by testing predetermined pixels in the output.

4. (2 points) Demonstrate by simulation of a VHDL test bench that the Line_Rasterization module is capable of generating an interpolated line of points by outputting the appropriate pixels.

6. Appendices

A. Output animations for test assembly programs

B. Test assembly programs

C. VHDL schematics

D. VHDL test bench results

E. VHDL code

F. Test benches

G. Report files from entire design synthesis

H. Simulator code

A. Output animations for test assembly programs

2D Lines Sourced/Mapped Output

3D Cube Sourced/Mapped Output

B. Test Assembly Programs

2D TEST

xyfov	90

color	128

clr

rstmat

ln2d	0 0

128 128

color	192

ln2d	255 0

128 128

color	64

ln2d	128 128

0 255

color	96

ln2d	128 128

255 255

flip

3D TEST

xyfov	90

color	128

clr

rstmat

trans	0 0 4

yrot	0

ln3d	-1 -1 -1

-1 -1 1

ln3d	-1 -1 -1

-1 1 -1

ln3d	-1 1 1

-1 -1 1

ln3d	-1 1 1

-1 1 -1

ln3d	1 -1 -1

1 -1 1

ln3d	1 -1 -1

1 1 -1

ln3d	1 1 1

1 -1 1

ln3d	1 1 1

1 1 -1

ln3d	-1 -1 -1

1 -1 -1

ln3d	-1 -1 1

1 -1 1

ln3d	-1 1 -1

1 1 -1

ln3d	-1 1 1

1 1 1

flip

clr

rstmat

trans	0 0 4

yrot	5

……

C. VHDL Test Schematics

CONTROLLER.VHD Schematic

DIVIDE.VHD Schematic

DIVIDE_8BIT.VHD Schematic

LCD_CONTROLLER.VHD Schematic

SCREEN_BUFFER.VHD Schematic

SHIFT_REGISTER.VHD Schematic

SRAM_CONTROL.VHD Schematic

D. VHDL Test Bench Results

Line Rasterization Test Bench

Controller Test Bench

GPU Test Bench I

GPU Test Bench II

Line Rasterization Test Bench

Matrix Math Test Bench

Screen Buffer Test Bench

Shift Register Test Bench I

Shift Register Test Bench II

SRAM Control Test Bench

E. VHDL code

F. Test benches

G. Report files from entire design synthesis

H. Simulator code

0000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000