3DGPU

3D Graphics Processing Unit

Proposal

James Doty

Kyle McGhee

Allan Patterson

Daniel Wilhelm

ECE 495D

Tuesday, 2:30-5:20pm

TA: Philip Vorsilak

Submitted 03/11/05

___________________________  ___________________________

___________________________  ___________________________

Executive Summary

Graphics Processing Units (GPUs) have become standard components in modern-day computer systems, enabling users to render two- and three-dimensional graphics in parallel with the microprocessor. GPUs can render graphics efficiently due to the massive parallelization possible within the graphics pipeline, making it an ideal target for hardware speedup. Recently, several GPU manufacturers began developing low-power GPUs aimed at the burgeoning cell phone market, realizing increasing demand for user interfaces, screen savers, and games.

A 3D Graphics Processing Unit (3DGPU) capable of rendering 3D points and lines to a low-resolution, low-color-depth graphics display will be implemented, similar to those on modern high-end cell phones and handheld gaming devices.

The design is important because it forms the basis for a complete pipelined GPU. Alone, it can render all possible 3D entities given an appropriate CPU algorithm, including meshes and filled polygons. The 3DGPU also incorporates a z-buffer, allowing easy hidden surface removal. It is also unique due to a caching system which enables instruction batches to be rendered continuously without any CPU intervention. Future projects could easily extend this pipelined framework, incorporating support for more advanced primitives and algorithms as well as the parallelization support necessary to compete with modern cards.

To develop the 3DGPU, four engineers are required for six weeks. VHDL will be used to implement high-level pipeline stages such as 3D projection and line rendering, and C code will be written to test the design by simulating an LED display panel. Since the design is very modular, each module will be given to a separate engineer for initial development and testing.

This document contains the proposal for the 3DGPU project. In Section 2, general design specifications are presented along with the available opcodes that the processor will support. Section 3 contains a top-level block diagram of the 3DGPU, illustrating input and output pins along with output signals. Section 4 illustrates our projected milestones and task assignments amongst the engineers, and Section 5 concludes the proposal by giving criteria that our product can be tested on that ensure success.

2.1. Introduction
	Graphics Processing Units (GPUs) are microprocessors which are optimized for the display of 3D graphics and 2D rasterization by parallelizing each block in the graphics pipeline in hardware. As computers have undergone miniaturization, now being available in cell phones and handheld videogame consoles, consumers have demanded more features in smaller areas. Companies such as nVidia and ATI have recently responded to this demand by developing low-power GPUs which provide 2D and 3D acceleration to these devices while dealing with constraints such as reduced functionality, fewer pins, and lower power dissipation.

The purpose of the 3DGPU project is to develop a bare-bones GPU designed for embedded devices such as cell phones that is capable of accelerating 2D and 3D graphics rendering. The project also will provide advanced features such as page flipping to reduce flicker and z-buffering for hidden surface removal. From the user’s perspective, all 2D and 3D operations will be rendered faster on their display. If the 3DGPU is programmed correctly, the user will notice that flicker does not occur and that pixels on the screen are ordered by their depth away from the user such that closer objects occlude farther objects.

The 3DGPU application is a hand-held device designed to operate from a low voltage power source. The primary special requirements for the 3DGPU are power dissipation and pin count since we are aiming our device to be eventually used for larger next-generation high-end cell phones and low-end handheld videogame consoles. Since miniaturization of transistors yields more power leakage, more powerful cooling devices are required as more power is leaked. However, since we are designing for larger, more high-end video quality models, leakage can be slightly more generous. Pin count is also an important factor because for each pin, the output logic for that pin must be localized next to it; thus, the more pins used, fewer area optimizations are possible, and also the distance between each pin is reduced, increasing the possibility of leakage between pins.

The primary optimization objective of the 3DGPU is speed. GPUs are often measured by the number of elements rendered per unit time, and this is directly dependent on speed and the algorithms used. For instance, the 3DGPU will thus require an efficient parallelized line algorithm since that module is the largest bottleneck in the pipeline. More importantly, since the 3DGPU will serve as the foundation for more advanced GPUs, it must be as fast and transparent a layer as possible.

Queue/Controller

Matrix Math

3D Projection

X FOV (latched)

Y FOV (latched)

Line Rasterization

Screen Buffer

currColor (latched)

currPage (latched)

SRAM

LCD Controller

(external)

LCD Display

(external)

CPU

Instruction

Graphics Pipeline (Periphery)

Vector

Data

Matrix

Data

CurrMatrix

CPU Read/Write

Video Buffers

ROW/COL Signals

3D Point

3D Point

Two 2D Points + Z

2D Point + Z

2D Point + Color

QTOP (latched)

QBOT (latched)

QPOS (latched)

Vector (ie 3D Point)

addr

addr

Matrix (4x4)

FIGURE 1.

3DGPU high-level block diagram.

2D Point + Color + Z

2D Point + Color

3DGPU

2.2 DESIGN CONSIDERATIONS

op addr1 addr2

3DGPU System Inputs (From the CPU)

A CPU exists externally to the GPU. The CPU can read and write directly to any area of the SRAM. An output signal from the SRAM module (mem_wait) is high when the memory is being accessed. Any device requesting to read or write to SRAM must wait until mem_wait is cleared before proceeding to ensure data integrity. The CPU also sends an assembly instruction to the 3DGPU consisting of a 4-bit opcode and two 16-bit addresses which, according to the opcode, either reference absolute addresses in SRAM or short integer values. Fed in by singular pins from the CPU, there is a load flag pin, LOAD, which loads an instruction into the queue, as well as a clock pin CLK and an asynchronous reset pin nRST.

3DGPU System Outputs (to the LCD Display Driver)

The GPU uses these inputs to drive the eight-bit LCD_Out bus, which connects to an LCD Driver Chip whose output routes to an LCD Display. Each 8-bit pixel color value is outputted serially, starting at the upper-left pixel of the display, proceeding to the end of a scan line, then down to the lower-left of the next row. The internal queue will also assert a single pin QUEUE_FULL signal back to the CPU if the queue reaches an out of bounds address.

3DGPU Data Formats

The 3DGPU contains several important data formats, illustrated in Table 1.

3DGPU Operations

Each of the 3DGPU operations consists of a series of control signal (CMD_*) transfers from Controller to each Module (Tables 2 and 3), along with an accompanying START_* signal to the appropriate module. Upon completion, each module returns a DONE_* signal back to the Controller

‘setmat’ - Controller passes address into matrix_math on ADDR_MATRIX.  Matrix_math then retrieves the matrix from the location pointed to by addr, and sets it as the module’s current stored matrix.

‘mulmat’ – Controller passes address into matrix_math on ADDR_MATRIX.  Matrix_math then retrieves the current stored matrix, as well as the matrix pointed to by the input addr, multiplies the two matrices, and stores the result in the new current stored matrix.

‘rstmat’ – Controller accesses matrix_math, and matrix_math loads the current stored matrix with the identity matrix.

‘xfov’ – Controller passes a value through VECTOR_X to in_vector_x to matrix_math.  Matrix_math passes this value without modification to 3D_Projection into Vector_3d_x. 3D_projection converts the value from floating-point to fixed-point, and then stores the value, by latching, in the module.

‘Yfov’ - Controller passes a value through VECTOR_Y to in_vector_y to matrix_math.  Matrix_math passes this value without modification to 3D_Projection into Vector_3d_y. 3D_projection converts the value from floating-point to fixed-point, and then stores the value, by latching, in the module.

‘pt3d’ – Controller passes two address serially into matrix_math by ADDR_MATRIX. Matrix_math loads the current stored matrix from SRAM, multiplies the input by the vector passed in to matrix_math by the Controller via VECTOR_X, VECTOR_Y, VECTOR_Z, which were obtained by the Controller from SRAM at the start of the instruction.  The new vector is passed by out_vector_x, out_vector_y, out_vector_z into 3D_projection.  The input x and y values are then divided by the FOV constants, and then divided by the z value.  These scaled values are output by vector_2d_x, vector_2d_y, vector_2d_z to line_rasterizer. Line_rasterizer then waits until the process is repeated for the identical second input vector, from the Controller.  Once both values are loaded, line_rasterizer creates a 2D point, which is passed by out_vector_x, out_vector_y, out_vector_z, and asserted by Screen_Draw to the screen_buffer, along with the DONE_LINE signal back to the Controller.  Once the Buffer draws the signal, the DONE_SCREEN signal is returned to the Controller.

‘ln3d’ – Controller passes two address serially into matrix_math by ADDR_MATRIX. Matrix_math loads the current stored matrix from SRAM, multiplies the input by the vector passed in to matrix_math by the Controller via VECTOR_X, VECTOR_Y, VECTOR_Z, which were obtained by the Controller from SRAM at the start of the instruction.  The new vector is passed by out_vector_x, out_vector_y, out_vector_z into 3D_projection.  The input x and y values are then divided by the FOV constants, and then divided by the z value.  These scaled values are output by vector_2d_x, vector_2d_y, vector_2d_z to line_rasterizer. Line_rasterizer then waits until the process is repeated for the second input vector, from the Controller.  Once both values are loaded, line_rasterizer generates a sequence of interpolated 2D points, which are passed one at a time by out_vector_x, out_vector_y, out_vector_z, and asserted by Screen_Draw to the screen_buffer.  Once the Buffer draws the signal, the Screen_Done signal is returned to line_rasterizer, and the next point is sent to the screen_buffer.  Once the end of the line is reached, line_rasterizer asserts the DONE_LINE signal back to the controller.

‘color’ – The Controller sets the VECTOR_COLOR value, passes it without modification through matrix_math, 3D_projector, and line_rasterizer, and finally into screen_buffer by vector_color.  The screen_buffer module locally stores the value by latching it.

‘clr’ – The Controller passes a control signal straight to screen_buffer.  Screen_buffer uses the current latched color to write to the current video page in screen_buffer, and fills the entire latched current video buffer with that color.

‘flip’ – The Controller passes a control signal straight to screen_buffer.  Screen_buffer toggles between the active video page and the inactive video page by inverting a local variable.

‘pt2d’ – A vector is loaded into the Controller via VECTOR_X, and VECTOR_Y; the value of the VECTOR_Z coordinate is forced to 0 for this 2D operation by the controller.  The new vector is passed straight until screen_buffer without modification. The value is loaded into screen_buffer, along with the DONE_LINE signal being sent back to the Controller.  Once the Buffer draws the signal, the DONE_SCREEN signal is returned to the Controller.

‘ln2d’ – Two vectors are loaded in serial to the Controller via VECTOR_X, and VECTOR_Y; the value of the VECTOR_Z coordinate is forced to 0 for this 2D operation by the controller.  The first new vector is passed straight until line_rasterizer without modification. Once this vector is loaded into line_rasterizer, the DONE_LINE signal is sent, and the second vector is passed into line_rasterizer.  A series of interpolated values are passed into screen_buffer, along with the SCREEN_DRAW signal.  Once each value is drawn by the screen_buffer, the SCREEN_DONE signal is returned to line_rasterizer.  Once the line is completed, the DONE_LINE signal is sent back to the Controller.  Once the Buffer draws the final signal, the DONE_SCREEN signal is returned to the Controller.

‘clrq’ – The controller internally sets the current queue position (qpos) to the top of the queue (qtop) (Table 5).

‘Nop’ – Do absolutely nothing except waste a clock cycle.

| Data Type | Description | Bytes | C data storage type |
| --- | --- | --- | --- |
| INT | Integer input from CPU | 4 | unsigned long |
| FLOAT | Floating Point (IEEE) input from CPU | 4 | float |
| addr | Absolute SRAM Address input from CPU | 2 | short int |
| FPOINT | Fixed Point (IEEE) to store internal data | 4 | unsigned long |
| COLOR | Pixel Color 8-bit color values | 1 | unsigned char |
| 2DVEC | 2D Vector fixed point values | 8 | struct 2DVEC { FPOINT x, y; }; |
| 3DVEC | 3D Vector fixed point values | 12 | struct 3DVEC { FPOINT x, y, z; }; |
| MATRIX | 4x4 Matrix fixed point values | 64 | struct MATRIX { FPOINT x1, y1, z1, w1; FPOINT x2, y2, z2, w2; FPOINT x3, y3, z3, w3; FPOINT x4, y4, z4, w4; }; |

TABLE 1. Data Formats Used.

| Command Signal | Matrix Math | 3D Projection | Line Rasterization | Screen Buffer |
| --- | --- | --- | --- | --- |
| 000 | pass | pass | pass | no draw |
| 001 | vector multiply and pass | project and pass | rasterize and pass | set color |
| 010 | store in SRAM | set X FOV |  | plot pixel |
| 011 | store identity | set Y FOV |  | clear buffer |
| 100 | Matrix multiply and store |  |  | page flip |
| 101 |  |  |  |  |
| 110 |  |  |  |  |
| 111 |  |  |  |  |

TABLE 2. Module Control Codes. 
NOTE: defaults to ‘pass’ if undefined.

| Opcode | Mnemonic | Matrix Math | 3D Projection | Line Rasterization | Screen Buffer |
| --- | --- | --- | --- | --- | --- |
| 0000 | setmat | 010 |  |  |  |
| 0001 | mulmat | 100 |  |  |  |
| 0010 | rstmat | 011 |  |  |  |
| 0011 | xfov |  | 010 |  |  |
| 0100 | yfov |  | 011 |  |  |
| 0101 | pt3d | 001 | 001 |  | 010 |
| 0110 | ln3d | 001 | 001 | 001 | 010 |
| 0111 | color |  |  |  | 001 |
| 1000 | clr |  |  |  | 011 |
| 1001 | flip |  |  |  | 100 |
| 1010 | pt2d |  |  |  | 010 |
| 1011 | ln2d |  |  | 001 | 010 |
| 1100* | clrq |  |  |  |  |
| 1101* | nop |  |  |  |  |
| 1110 |  |  |  |  |  |
| 1111 |  |  |  |  |  |

TABLE 3. Control Code Transition Chart By Opcode
NOTE: defaults to ‘pass’ if undefined.
* indicates opcode which does not utilize any module outside the controller or SRAM

| OPCODE | MNEMONIC | DESCRIPTION |
| --- | --- | --- |
| 0000 | setmat addr_MATRIX | Set Current Matrix. |
| 0001 | mulmat addr_MATRIX | Multiply Current Matrix. |
| 0010 | rstmat | Reset Current Matrix to Identity. |
| 0011 | xfov val_FLOAT | Set X Field of View. |
| 0100 | yfov val_FLOAT | Set Y Field of View. |
| 0101 | pt3d addr_3DVEC | Draw 3D point. |
| 0110 | ln3d addr_3DVEC addr_3DVEC | Draw 3D line. |
| 0111 | color val_INT | Set Color. |
| 1000 | clr | Clear Buffer. |
| 1001 | flip | Page Flip. |
| 1010 | pt2d addr_2DVEC | Draw 2D point. |
| 1011 | ln2d addr_2DVEC addr_2DVEC | Draw 3D line. |
| 1100 | clrq | Clear queue. |
| 1101 | nop | No Operation. |
| 1110 |  |  |
| 1111 |  |  |

TABLE 4. Supported Feature Set
NOTE: Defaults to ‘No Operation’ if undefined.

‘addr_{TYPE}’ indicates that an absolute address is passed as an operand that points to the type TYPE in SRAM.

‘val_{TYPE}’ indicates

| Storage Variable | Bytes | Initial Value Upon Reset | Location |
| --- | --- | --- | --- |
| X FOV FPOINT | 4 | 1 (note: 90 degree FOV) | 3D Projection |
| Y FOV FPOINT | 4 | 1 (note: 90 degree FOV) | 3D Projection |
| currColor COLOR | 1 | 0 (black) | Screen Buffer |
| currMatrix MATRIX | 64 | Identity | SRAM |
| Video Page 1 | (display width x display height) x (depth bytes) | Reset to 0s | SRAM |
| Video Page 2 | (display width x display height) x (depth bytes) | Reset to 0s | SRAM |
| currPage | 1 bit | 0 | Screen Buffer |
| QTOP | 1 | Reset to constant value | Queue/Controller |
| QBOT | 1 | Reset to constant value | Queue/Controller |
| QPOS | 1 | Reset to QTOP | Queue/Controller |
| Queue | TBD | Reset to 0s | SRAM |

TABLE 5. Stored/Initial Values