Part 1: Introduction

For this ECE 362 Mini-Project Design, our team developed a Memory Game using a Motorola 68HC12B32 microprocessor, its associated peripherals, and eight BCD-encoded 7-segment LED displays.  The object of the game is to match the pairs of the numbers one through four (1-4) that are placed randomly on the LED displays and hidden from view at the start of the game. When a pair is selected together, the numbers remain displayed until the game is completed. It is a reproduction of the classic game “Concentration.”

Two 10k potentiometers are used to initialize a random sequence with which to insert the numbers on the LED displays. Input from the user is taken from the keyboard.

Part 2: Interfacing

The Digital Sliding Block Puzzle was developed and implemented using a variety of digital resources.

The following devices were used to create and form the Digital Sliding Block Puzzle:

(1)	Motorola 68HC12 microprocessor, P&E Micro board, & development software

(8)	7-segment LED displays

(8)	74LS47 BCD to 7-segment Decoder/Drivers

(3)	Solder-less Breadboards

(1)	5V DC power supply

(56)	150 ohm resistors

(1)	TeraTerm COM port interface software package

The output ports on the Motorola 68HC12 microprocessor were programmed to run in “Normal Drive Mode” as the 74LS47 decoder chips are TTL logic based.  Each 74LS47 decoded its specific display signal for its corresponding 7-segment LED display.

Each nibble (4 bits) of Ports A, B, P, and T were used to drive the game display for a total of 8 nibbles, each corresponding to one BCD Decoder/7-segment display “group.”

Part 3: Peripherals

To challenge the game player to improve his or her speed at winning the game, a running game clock was implemented to give an accurate time taken to complete each game.  Using the Real-Time Interrupt module (RTI) on the microprocessor, an interrupt-driven game clock was created.  The RTI was programmed to update the game clock, which is displayed upon completion of the game.

The serial communications interface (SCI) and its standard transmit/receive pins via a serial port on the P&E Micro development board were used to interface with the COM port on the lab station PC.  This allowed for the creation of a visual interface using the TeraTerm software package as well as a means for obtaining user input (from the numerical keypad on the keyboard).  All user-level game settings and game play occurred via the SCI.  The SCI peripheral was developed as a completely program-driven process.  To provide the game clock with a higher level of accuracy, the SCI peripheral implementation was developed as a completely program-driven process.

Part 4: Software

The following is a listing and brief description of each software module that was written to implement the Digital Sliding Block Puzzle:

- Start – register initializations

- main –  visual interface initialization, user-level inputs, & game play control.

- init_all – calls all other initialization routines

- compare – compares each guess of the round. If they are equal, calls match, otherwise calls wrong.

- wrong – turns off the LEDs that were guessed.

- match – increases “correct” count and leaves LEDs lit.

- win -- diplays winning message and time

- welcome – welcomes the user

- blankit  -- blanks the display

- set_puzz – initializes the LEDs (the “board”) based on the ATD potentiometers

- offLED – turns off  the correct LED

- onLED – displays the correct LED

- Set_LED – will ROL the correct memory for correct display

- inatd – samples the ATD potentiometers

- delay – provides 3 seconds of delay

- rti_isr – counts till 1 second has passed

- clock – increments clock value by one second

- upmin – increments clock value by one minute

- cdisp – displays clock value on the screen

- backup – backs up the number of spaces passed in the B register

- init_ports – initializes the output ports to general output

- init_sci – initializes asynchronous serial port (SCI) for 9600 baud rate

- init_rti – enables RTI for 8.192 ms interrupt rate

- init_atd – enables ATD for program-driven interrupts. Receives two potentiometers on Channels 1 and 2

- Auxiliary functions (self-explanatory)

  - inchar

  - outchar

  - pmsg

  - atoh

  - htoa

Part 5: Summary & Conclusions

The entire creation, design, development, and implementation of the Memory Game for this ECE 362 Mini-Project proved successful.  From its inception, this project challenged its creators to utilize all of the necessary digital and electronic, as well as organizational resources to create a successful and fun product.

There was not an ungodly amount of time spent on this design, but the time spent was well-applied.  By taking the time and effort to properly organize and develop this project idea, as well as structure the assembly-code modules to handle simple and efficient tasks, the project development flowed smoothly and quickly.  Very few setbacks came from debugging as a result of preliminary efforts made to keep the code organized and readable.  This allowed the creators to spend more time developing a creative, fun, and imaginative project that ended up delighting all who viewed it.  More so, the project was tremendously successful with respect to the original design goals.

Initially, we had attempted to interface the M68HC12B microcontroller with a stage light, using 3 potentiometers to control various features of the light. The majority of our time spent developing this mini-project was spent in designing and coding for this stage light. This, however, proved to be unfeasible, as we could not reach the necessary baud rate using the SCI. While an alternative solution was available by rapidly switching port bits, we decided to switch to the game Concentration after realizing that with a 250 Kb baud rate required, there were few practical ways to debug improper operation.

Part 6: List of References

- ECE 270 Course Website: Lab Experiments and Homework: Experiment 7: Document

http://shay.ecn.purdue.edu/~dsml/ece270/Labs/PDF/2-Lab07.pdf

- ECE 362 Lecture Notes: Modules 3-5

http://shay.ecn.purdue.edu/~dsml/ece362/Notes/index.html

- SN74LS47 Data Sheet

http://shay.ecn.purdue.edu/~dsml/ece270/Refs/Chips/7447.pdf

- Motorola Semiconductor Technical Data – CPU12 Reference Guide

http://shay.ecn.purdue.edu/~dsml/ece362/Refs/Hc12/cpu12rg.pdf

- Motorola Semiconductor Technical Data – CPU12 Reference Manual

http://shay.ecn.purdue.edu/~dsml/ece362/Refs/Hc12/Cpu12ref/index.html

- Motorola Semiconductor – M68HC12B Family Data Sheet

http://shay.ecn.purdue.edu/~dsml/ece362/Refs/Hc12/M68HC12B.pdf

Appendix A: Activity Logs

Appendix B: Solution Source Files
(1) Memory Game Source

(2) DMX Stage Light Controller Source

Appendix C: Circuit Schematic & Photos